# Setup Flask Server to Run the GoPiGo3

## Connect to the GoPiGo3

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Use these instructions to install Raspbian for Robots on your GoPiGO3: https://www.dexterindustries.com/howto/install-raspbian-for-robots-image-on-an-sd-card/ | The Raspbian for Robots OS is installed on the GoPiGo3 |
| 2 | Use these instructions to setup the wifi on your GoPiGo3: https://www.dexterindustries.com/GoPiGo/get-started-with-the-gopigo3-raspberry-pi-robot/2-connect-to-the-gopigo-3/raspbian-for-robots-operating-system/ | The GoPiGo is setup to connect over wifi |
| 3 | Open a browser and enter http://dex.local | The Raspbian for Robots webpage is displayed |
| 4 | Click Launch VNC | The browser connects to the GoPiGo GUI |

## Setup Python Virtual Environment

- Open file manager and create a new folder

- Open a terminal and navigate to the new folder

- Create a new python virtual environment with the command “python3 -m venv venv”

- Activate the virtual environment with “. venv/bin/activate”

- Upgrade pip with “pip install --upgrade pip”

Note: If pip displays SSL errors use this command to ignore:

$ pip install --trusted-host pypi.org --trusted-host files.pythonhosted.org pip setuptools

- Use pip to install these packages:

  - pip install Flask

  - pip install opencv-python

  - pip install gopigo3

- download the project files from the gitLab repository

- Copy all of the files from the “WorkingProject” folder into the folder where your virtual environment is setup on you GoPiGo3. This can be done with a USB drive or using the scp command

  - To use the scp command open a terminal on your computer and enter:

scp –r <source > <destination>

  - The files should now be copied to the “NewProject” folder on your GoPiGo3

- Start the Flask server by entering “python app.py”

Note: If a libcblas.os.3 cannot open shared object error is thrown, run these command:

pip3 install opencv-python

sudo apt-get install libcblas-dev

sudo apt-get install libhdf5-dev

sudo apt-get install libhdf5-serial-dev

sudo apt-get install libatlas-base-dev

sudo apt-get install libjasper-dev

sudo apt-get install libqtgui4

sudo apt-get install libqt4-test

- On your computer open a browser and navigate to http://dex:5000 or http://<gopigo IP>:5000